INF2004 Embedded Systems Programming

# IS29 Pre Tribunal 1

Architecture Defence & Integration Readiness | AI ideation, live update, AI-vetted gap
check

# Team Information

| Item | Details |
| --- | --- |
| Group ID | IS29 |
| Repository Link | https://github.com/teck1341/ICT_INF2004_Embedded.git |

| Buddy | Student ID | Primary subsystem / persona |
| --- | --- | --- |
| Buddy 1 Subque | 2500599 | Network Gatekeeper: WiFi, MQTT, telemetry |
| Buddy 2 Ong Teck Ang | 2500738 | Real-Time Taskmaster: motors, encoders, PID |
| Buddy 3 Tan Yi Jian Ethan | 2501528 | Track Purist: IR sensors, line following, barcode |
| Buddy 4 Dian Ze | 2500916 | Physics Watchdog: IMU, terrain, hump/motion events |
| Buddy 5 Siang Hong | 2503147 | Spatial Optimizer: ultrasonic, servo, obstacle profiling |

# Pre-Tribunal AI Ideation

These completed critique rows use the five recorded AI sessions commissioned by the coordinator on 25 September 2026 and their source-checked reviews in Part1_Week6_Report/AI_Sessions, also summarised in report_entries.json. The final column is a prepared Version A position updated for the Week 6 feedback on 15 October. These are existing AI discussions, not five newly conducted buddy sessions; individual endorsement, implementation and hardware validation are not asserted.

| Buddy | AI prompt used | Useful AI ideas | Weak / unrealistic AI ideas | Position before Zoom |
| --- | --- | --- | --- | --- |
| B1 — Subque Communications | Recorded prompt: critique MQTT telemetry and remote STOP; identify failures that lose/delay STOP or starve control, and rank practical improvements. | Repeat STOP until the status stop flag is seen; treat 1 s of telemetry silence as remote STOP unavailable and use GP20. Test reception and radio progress under combined load. | Reject command sequence gating: restarting the dashboard could make its STOP look stale. A repeated STOP is harmless. A status round trip cannot prove the internal stopping deadline. | Retain exact STOP tokens and local GP20. Propose the repeat/silence procedure (T-02/T-35). Show actual P1–P8 application priorities and P9 radio ownership; source configuration already has a 1 ms tick and absolute releases. Measure jitter, radio progress and reception-to-brake latency (T-36); split FR9 into schema, cadence, liveness and reconnect tests. |
| B2 — Ong Teck Ang Motion | Recorded prompt: review the 5 ms wheel controller, 50 ms encoder-speed window, motion leases, stall detection and STOP at motor commit; identify accuracy and safety risks. | Measure each wheel's duty/speed and breakaway curve before adding feedforward. With placeholder gains, calculated startup to 0.15 duty takes about 3 s; this can conflict with stall detection. | Reject simply softening lease expiry: Vehicle staleness faults after 40 ms, before the 50 ms lease. Expiry with a fresh supervisor indicates a separate defect; a soft label alone adds no safe recovery. | Keep one PWM owner and immediate braking. Measure counts/revolution, wheel geometry and stopping distance (T-30/T-31), then tune feedforward/stall thresholds (T-05/T-59). Use 0.08 m/s as the provisional operating speed; 0.15 m/s remains a cap. Separate hazardous faults from bounded mission retries, and verify primitive timeouts against distance/speed before increasing speed. |
| B3 — Tan Yi Jian Ethan Optical navigation | Recorded prompt: critique IR line following, encoder-derived barcode widths and the next-junction 60 mm approach followed by a pivot; identify likely failures and practical fixes. | End a junction pivot on the leading line sensor's black edge, with encoder yaw as a sanity bound. Straight REACQUIRE cannot reliably remove heading or lateral error after an inaccurate turn. | Reject automatic 180 mm reversal to reread rejected frames: junctions can also reject frames, and there is no rear ranging or reverse line following. A 2 ms nominal period alone does not prove barcode resolution. | Propose sensor-ended turns and bounded lateral reacquisition with B5 (T-31/T-52). Validate actual counts/mm and worst sample gaps at 0.08 m/s (T-06/T-30/T-36). Define a labelled practice codebook and distinct completion event: brake, publish completion and inhibit navigation until reset. Confirm official symbols later; quantify FR1 lighting, geometry, speeds and trial criteria. |
| B4 — Dian Ze Terrain and hump height | Recorded prompt: critique GY-511 accelerometer-only tilt, collision detection, motion classification and path-times-sine height; identify real-track errors and justified changes. | The two-contact chassis geometry biases the height estimate: an illustrative 30 mm-high, 100 mm-long triangular hump with 110 mm contact spacing gives about 14 mm. This is a model calculation, not a trial result. | Reject raw encoder acceleration subtraction before pitch: one assumed encoder count over 10 ms corresponds to about 5.7 m/s². It can inject more error than it removes; windowed acceleration needs validation first. | Run hump-crest, braking and pivot false-alarm trials before changing hard-fault thresholds (T-33/T-67). Window deceleration (T-53), measure contact spacing and compare a two-contact model with ground truth (T-08). Keep the present estimate explicitly labelled until the assessed height definition and T-22 revision are settled. Measure normal and timeout I2C load; propose stopped, bounded recovery for transient sensing failures. |
| B5 — Siang Hong Obstacle handling and integration | Recorded prompt: stress-test the ±60° sonar scan, side choice, bypass and reacquisition; identify failure modes and practical fixes. | Calibrate sonar beam-width inflation: the illustrative 15° cone adds about 80 mm at 0.30 m. Correct reported width only; retain conservative raw geometry for clearance. Replace straight-only reacquisition with a bounded line-crossing search. | Reject pointing the only sonar sideways while driving to infer obstacle depth: forward guarding is lost and oblique faces can imitate an edge. Reject treating missing echoes as clear space. | Prioritise a measured scan/turn/travel/reacquisition budget (T-50/T-65) and lateral search (T-52); 31–36 s remains an estimate, not sub-10 s compliance. Propose stopped retries with explicit attempt/time/distance limits. Permit reverse-and-reattempt only with established rear clearance. Split FR7/FR8 into testable outputs, and show the input supply rail and ECHO divider in the wiring table. |

# Week 6 Architecture Defence Snapshot Before Zoom

Six architecture claims are defended below against source evidence, remaining weaknesses and the responsible buddies.

| Week 6 artefact | What the team claimed in Week 6 | Evidence available now | Still weak or untested | Buddy defending it |
| --- | --- | --- | --- | --- |
| Safety and Motion | Only Motion commits drive PWM; a fault can brake. Leases and freshness checks inhibit stale control. | Source: robot_app.c, motion.c. Historical application checks in E1. | Target latch-to-output latency and physical stopping unmeasured; recovery too restrictive. | B2 B5 |
| RTOS task table | Safety has priority over control; application tasks share one core. | Source already uses numeric P1 to P8, radio P9, 1 ms installed tick and absolute releases. | No measured WCET, jitter, stack high-water or radio progress under worst load. | B1 B2 |
| Optical and routing | Encoder widths support barcode actions at a later junction. | line_ctrl.c, barcode.c, course_cfg.c; host checks in E1. | Empty production codebook, unresolved finish, actual encoder resolution and barcode gap budget. | B3 |
| Terrain | Filtered acceleration and signed path estimate pitch and hump height. | terrain.c; Week 6 method and host checks. | No gyro; false hard faults and two-contact height bias need trials. | B4 |
| Scan and bypass | Unknown range never means clear; scan feeds bounded bypass and reacquisition. | scanner.c, vehicle.c; 65 ms sonar trigger interval. | 31 to 36 s estimate exceeds under-10 s band; no lateral search or reverse-and-reattempt. | B5 B3 |
| Communications | One radio owner processes MQTT; Comm consumes commands and telemetry is bounded. | net_mqtt.c; install_network.py injects polling in radio task. | Remote STOP best effort; radio outside watchdog gate; live network unverified. | B1 |

## Shared Resource Allocation Matrix

The matrix identifies shared resources, owners, access rules, failure modes and the evidence needed to validate them.

| Shared resource | Used by task(s) | Owner task / module | Access type | Contention risk | Mitigation | Evidence needed |
| --- | --- | --- | --- | --- | --- | --- |
| Motor PWM | Motion; Safety; Vehicle | Motion B2 | Commit or brake | Unsafe late write | Fault recheck in locked commit; others may only brake | STOP during commit test |
| GPIO ISR and counts | Encoder; ECHO; Motion; Line; Scan | board.c B2 B5 | IRQ and snapshots | Lost edges; long masked time | One ISR; short PRIMASK copies | IRQ and copy duration |
| I2C1 and CPU | IMU; all runnable tasks | IMU B4 | Busy-wait read | Timeout blocks lower priorities | Bound timeouts; measure execution; assess split read | Normal and timeout load |
| Radio and IPC | Radio; Comm; Telemetry | Radio B1 | Queues and lwIP | Starvation; drops | Only P9 calls network; bounded IPC | Liveness and backlog log |
| Shared state and route | Vehicle; Motion; Line; Scan; IMU | Named producer per field | Locked snapshots | Stale data; conflicting action | Units, validity, age and completion IDs | Stale and duplicate injection |
| PWM slices and power | Motors; servo; sonar | board.c B2 B5 | Outputs and rail | Slice conflict; wrong V+ | Confirm mapped channels; measure rail and divider | Wiring photo and voltage |

## Subsystem Interface and Code Structure Map

Each buddy has an interface entry identifying providers, consumers, input/output contracts and validation.

| Module / file / interface | Provider | Consumer(s) | Purpose | Inputs | Outputs | Interface rule | Validation rule |
| --- | --- | --- | --- | --- | --- | --- | --- |
| net_mqtt IPC | B1 radio | Comm; telemetry | Network bridge | STOP; frames | Command; status | Network calls only P9; SI conversion at edge | Outage and saturation |
| motion.h | B2 Motion | Vehicle; IMU; Scan | Control and pose | Leased request; counts | m, m/s, rad; ID | One PWM writer; 50 ms lease | Expiry and STOP race |
| line_ctrl.h barcode.h | B3 Line | Vehicle | Line and route | ADC; counts | Error; action | No motor writes; one pending action | Speed and junction tests |
| terrain.h | B4 IMU | Safety; Vehicle | Terrain events | Accel; signed path | Tilt; height; validity | 100 ms freshness; units rad and m | Hump and brake trials |
| scanner.h | B5 Scan | Vehicle | Front and profile | ECHO; pose; rest | Range m; validity | 150 ms age; unknown unsafe | No echo and movement |

## Week 6 Assumption Closure

The assumption register covers all five buddies. Open status means the supplied evidence does not yet validate the assumption.

| Week 6 assumption ID | Original assumption | Current status | Evidence collected | Remaining risk | Next action |
| --- | --- | --- | --- | --- | --- |
| A1 B1 | Lab Wi-Fi and broker; command policy | Open | Software paths only | Remote STOP or start policy | Run outage/load trial; confirm policy |
| A2 B2 | Encoder mode; 360 counts/rev; geometry | Open | Config defaults | Distance and barcode error | Count edges; measure wheels and track |
| A3 B3 | IR resolves bars at 0.08 m/s | Open | Budget check only | Undersampling; lighting | Measure counts/mm and worst gaps |
| A4 B3 B2 | Codebook; junction; 60 mm approach | Open | Empty course_cfg.c | No end-to-end mission | Agree practice rule; confirm official map |
| A5 B4 | Flat IMU; path times sin pitch height | Open | Algorithm only | Contact bias; false fault | Six-face and ground-truth hump tests |
| A6 B5 | Obstacle depth and clear corridor | Open | 31 to 36 s estimate | Slow bypass; missed line | Measure geometry and bounded search |
| A7 all | Periods and stacks fit one core | Open | Static build and source | CPU overload; stack exhaustion | WCET, jitter, high-water and radio log |
| A8 B5 | GP20 and GP21 active low | Open | Configured pull-ups | Wrong STOP polarity | Verify before enabling motors |