INF2004 Embedded Systems Programming

Week 8 Tribunal 1 Immediate

Architecture Defence & Integration Readiness | AI ideation, live update, AI-vetted gap check

# Team Information

| Item | Details |
| --- | --- |
| Group ID | IS29 |
| Repository Link | https://github.com/teck1341/ICT_INF2004_Embedded |

| Buddy | Student ID | Primary subsystem / persona |
| --- | --- | --- |
| Buddy 1 Subque | 2500599 | Network Gatekeeper: WiFi, MQTT, telemetry |
| Buddy 2 | 2500738 | Real-Time Taskmaster: motors, encoders, PID |
| Buddy 3 | 2501528 | Track Purist: IR sensors, line following, barcode |
| Buddy 4 | 2500916 | Physics Watchdog: IMU, terrain, hump/motion events |
| Buddy 5 | 2503147 | Spatial Optimizer: ultrasonic, servo, obstacle profiling |

## Buddy Contributions During Zoom

Each buddy must complete one row. The contribution must be a real technical challenge, defence, trade-off, or correction raised during Zoom, not only an attendance statement.

| Buddy | Key challenge, concern, or argument raised | Challenge received | Decision or compromise reached |
| --- | --- | --- | --- |
| Buddy 1 | MQTT payloads are named explicitly, so comms changes most. Telemetry was integer JSON (status ≤ 255 B every 200 ms; perception ≤ 252 B and diagnostic ≤ 144 B every 1 s, telemetry.h) and commands were ASCII tokens up to 32 B (mqttcmd.c). Argued for a packed binary wire format built with explicit shifts and masks, not C bit-field structs, because bit-field layout is implementation-defined and the laptop must decode it. Raised the read-modify-write race on the shared fault mask between the radio task (P9) and Safety (P1). | From Teck and Dian Ze: does the radio task's Wi-Fi-loss write wipe a stall or IMU bit, and who decides the field ranges you pack? From Siang Hong: 17 conditions do not fit a 16-bit mask. | Status 255 → 17 B, perception 252 → 17 B, diagnostic 144 → 18 B (calculated). 1-byte commands (3-bit code + 5-bit fixed pattern). Every mask write goes through fault_set()/fault_clear() inside app_main_lock(); the M0+ has no LDREX/STREX. WIFI_LOSS is a non-stopping status bit. Producers sign off ranges; B1 keeps the wire format and the team data representation table. |
| Buddy 2 | Motion and encoder data and ISR-shared data are named explicitly. Motion data was double or 32-bit: request 32 B, motion output 96 B, motion_t 600 B. The encoder ISR keeps int32 counters that Motion reads and clears under PRIMASK (board.c:285-302). Argued that width = range ÷ resolution, not range alone: the PID integrator needs ≥ 16 bits or integral action is lost. | From Subque: do you set stall and encoder bits from the ISR or the task? From Dian Ze: why keep doubles in the PID if every field must be minimum-bit? Is an 8-bit command id safe? | Fault bits are set by the Motion task inside robot_app_fault() under app_main_lock(); the ISR only flags edge errors. Request 32 → 6 B, snapshot 96 → 16 B, encoder ISR word 36 → 2 B + flags. Task-private accumulators (PID integrator, pose) keep full precision; only shared, stored and sent copies are packed — scope confirmed with the lecturer [mm:ss]. 8-bit id is safe: half-range compare plus 50 ms lease; host test for the 255 → 1 wrap. |
| Buddy 3 | Optical data pretty much all double. Darkness, steering, 32 bar widths per frame. 16-symbol codebook of double templates. sizeof on M0+: barcode_config_t 4,344 B, barcode_state_t 320 B. Inputs are a 12-bit ADC and 0.567 mm encoder counts, so 12-bit fixed point and count widths lose nothing the sensors resolve. The M0+ has no FPU. | From Teck and Siang Hong: can a narrow bar still be read if widths are whole counts? Can a junction edge be lost if b_junction_edge is packed with other flags? | The narrowest bar is ≈ 3.0 mm ≈ 5.3 counts at 360 counts/rev: the limit is encoder resolution, unchanged by bit width; Teck measures counts/rev. Widths become uint8 counts (320 → ~60 B). Every packed flag write stays under app_main_lock(), one flag word per producing task. OPTICAL is split into two mask bits. Narrow/wide codebook deferred until the symbology is confirmed. |
| Buddy 4 | Hump estimation is named explicitly. terrain_output_t holds pitch, roll, yaw rate and four heights as double (≈ 72 B), plus a 9-value motion enum. Motion state needs 4 bits, not the briefing's 3. Hump height cannot be 8 bits because the valid range is 5–300 mm. Packed pitch must saturate at the 0.60 rad tilt limit, never wrap. | From Subque and Teck: can the IMU timeout path still stall the core, and can a packed pitch wrap past 0.60 rad and read as level? | Pitch/roll 12-bit signed mrad, saturating; heights 9-bit mm (0–511); motion 4 bits; 6 one-bit flags (72 → ~12 B). Height maths stays double; only the stored and sent copy is packed. IMU, TILT, TILT_UNAVAILABLE and COLLISION become mask bits b9–b12 under the lock. The ~8 ms I2C timeout window is unchanged and will be measured with the masked time. |
| Buddy 5 | The scanner is the largest RAM user hit: 80 samples x 32B roughly equals 2.5KB of .bss for 15 bits of information each (scanner.h:15, 53-58). NaN cannot be packed, so unknown range needs an explicit code. The fault mask would need 17 bits (15 codes + WIFI_LOSS + NO_ECHO). | From Ethan and Teck: does an angle-indexed table change scan behavior? Does no echo stop the car? Can Scan and Vehicle overwrite each other’s packed ready bits? | 61-entry unit16 table indexed by (angle+60)/2 (2560 -> 122 B). Existing scanner tests must pass unchanged. LOCAL/REMOTE STOP move to a 2-bit stop-source field so the mask fits 16 bits . NO_ECHO is non_stopping, ‘unknown range never grants forward motion’ keeps the car safe. Ready bools become single-writer 2-bit completion counters. B% keeps the shared-word ownership table. |

## Live Decision Change Log

Enter at least 3 changes AND 3 confirmed no-change decisions from the Zoom discussion. Include the change type and evidence link or timestamp whenever possible. One row may state “No change” if the team defended the Week 6 design successfully.

| Original idea / Week 6 design | Updated decision | Change type | Reason for change | Evidence link / timestamp | Owner / follow-up |
| --- | --- | --- | --- | --- | --- |
| Telemetry is bounded integer JSON text; frames distinguished by key names; commands are exact ASCII tokens up to 32 B (telemetry.h, mqttcmd.c). | Packed binary frames: 4-bit header (version, frame type) then minimum-width fields, explicit shifts/masks, little-endian. Status 17 B, perception 17 B, diagnostic 18 B. 1-byte commands; STOP routing compares one byte (net_mqtt_ipc.c:121). | Major redesign | The requirement demands compact binary MQTT payloads. JSON spends up to 255 B on 17 B of information. Bit-field structs are unsafe on the wire (implementation-defined layout). | telemetry.h; mqttcmd.c; net_mqtt_ipc.c; [mm:ss] | B1: encoder, Python decoder/sender, round-trip tests |
| Fault reported as one first-fault code (robot_app_fault_t, 16 values); no Wi-Fi-loss or no-echo condition; separate fault paths per task. | 16-bit active-fault mask (layout table below) + unchanged 4-bit first-fault cause + 2-bit stop source. LOCAL/REMOTE STOP leave the mask; OPTICAL split into two bits; WIFI_LOSS and NO_ECHO are non-stopping. One fault_set()/fault_clear() helper under app_main_lock(). | Major redesign | Requirement asks for a shared bitmask including Wi-Fi loss and no echo. 15 codes + 2 new = 17 bits; stop requests are not faults. The mask alone loses which fault came first, needed for review. | robot_app.h:11-29; robot_app.c:775-791; [mm:ss] | B2 (fault owner) with all; B5 keeps bit layout |
| Encoder ISR keeps int32 deltas, uint32 totals/phases/edge-error count (36 B). Motion request 32 B; motion output 96 B of doubles (board.c:101-112, motion.h). | One 16-bit ISR word (6+6-bit deltas, 2+2-bit phases), saturating; 16-bit wrapping totals; 2 sticky edge-error bits. Request 48 bits (6 B); snapshot 124 bits (16 B). Task read-and-clear stays inside PRIMASK. | Simplified | Ranges: ≤ 6.6 counts per 25 ms at 0.15 m/s; validation limits 0.15 m/s and 1.0 rad/s. Clearing deltas while keeping phases is a read-modify-write. | board.c:285-302; motion.c:263-293; robot_app.c:437-464; [mm:ss] | B2; B3/B5 confirm widths |
| Darkness, steering and bar widths are double; codebook 16 × 32 double templates (4,344 B); 9 separate bools (line_ctrl.h, barcode.h). | 12-bit fixed-point darkness; thresholds precomputed in ADC counts; shared steering 8-bit; status 3 bits; widths uint8 counts; flags packed per producer under lock. Narrow/wide codebook (≈ 6 B/symbol) only if Code 39 is confirmed. | Simplified (codebook: Deferred) | 12-bit ADC and 0.567 mm counts are the real resolution; double was false precision and is software-emulated on the M0+. Code 39 is inferred from the sample, not stated. | line_ctrl.c; barcode.h; robot_config.h:76-81; [mm:ss] | B3; lecturer confirms symbology |
| terrain_output_t: pitch, roll, yaw rate, four heights as double (≈ 72 B). | Pitch/roll 12-bit mrad saturating at the 0.60 rad limit; heights 9-bit mm; motion 4 bits; completed humps 6 bits; 6 one-bit flags (≈ 12 B). Internal maths stays double. | Simplified | Valid hump range is 5–300 mm (8 bits cannot hold 300). A wrapped angle could read as level and miss a tilt fault. | terrain.h; robot_app.c:545; robot_config.h:156; [mm:ss] | B4; B1 uses widths on the wire |
| Scan samples: list of 80 × {int32 angle, enum, double, bool} = 32 B each; NaN means unknown; echo state/rise/pulse are three ISR-shared variables; two ready bools with two writers. | 61-entry uint16 table indexed by angle (2-bit state, 12-bit mm, 1-bit fine); explicit validity on every distance; one uint32 echo word; ready bools → single-writer 2-bit completion counters. | Major redesign | Angles are always even within ±60°, so the angle needs no stored bits. Integers have no NaN. A packed byte with two writers is a lost-update race. | scanner.h:15, 53-58; scanner.c:328-341; board.c:113-117; robot_app.c:413-414; [mm:ss] | B5; existing scanner tests must pass |
| Single radio owner at P9; telemetry every 200 ms; STOP on a separate priority buffer; local GP20 stop; QoS 0, 15 s keepalive. | No change. | No change | Packing changes the payload, not who sends or when. Smaller frames shorten P9 publish time, so starvation risk falls. | net_mqtt.c:25-31; robot_app.c:754-763 | B1 measures task time before/after |
| All task periods and priorities (Safety 2 ms P1, Motion 5 ms P2, Line 2 ms P3, Vehicle 10 ms P4, IMU 10 ms P5, Scan 2 ms P6, Comm 5 ms P7, Telemetry 200 ms P8). | No change. | No change | The requirement changes representation, not behaviour or timing. Smaller g_shared shortens every PRIMASK-masked snapshot copy. | app_main.c:135; robot_app.c:720-763 | All measure step time before/after |
| 32-bit millisecond/microsecond timestamps with wrap-safe subtraction. | No change. | No change | A timestamp is a shared time base, not a ranged value; the ranged intervals are config constants. Narrowing forces conversions at every comparison. | line_ctrl.c; robot_app.c | All; revisit if lecturer requires |
| Safety behaviour: stall policy (¦duty¦ ≥ 0.15, no count for 500 ms), tilt > 0.60 rad and collision latch, 'unknown range never means clear', 0.25 m front stop. | No change. | No change | Fewer bits do not change which conditions latch; saturation keeps limits reachable. | robot_config.h:120-156; vehicle.c:526-540 | B2, B4, B5 |
| Vehicle state machine, 9 states incl. RANGE_WAIT. | No change: kept 9 states, 4 bits (merge of RANGE_WAIT into SCAN rejected). | Rejected | RANGE_WAIT resumes FOLLOW; SCAN leads to BYPASS; they fault differently on a pending junction. A merge only moves the bit into a phase field. | vehicle.h:12-23; vehicle.c:516-540; [mm:ss] | B3 + B5 |

## Resource and Assumption Review

| Item challenged | Evidence presented | Decision made | Impact on other buddies |
| --- | --- | --- | --- |
| Is the RAM saving real? | Static buffers: MQTT IPC TX 1,040 B, RX 144 B, pending 257 B; g_scanner ≈ 2.5 KB; barcode_config_t 4,344 B; motion_t 600 B; terrain state 248 B (sizeof/struct layout). | Report .bss from the arm-none-eabi map file before and after. Stack savings count only if APP_MAIN_STACK_BYTES (4,096) is lowered after measuring each task's high-water mark. | All five tasks copy g_shared onto their stack; lowering the stack needs every buddy's high-water figure. |
| Shared packed fault mask: read-modify-write race | Safety P1 (2 ms), Motion P2, Line P3, IMU P5, Scan P6 and radio P9 all raise faults; a bit-field write rewrites the whole word; M0+ has no LDREX/STREX. | Single fault_set()/fault_clear() helper inside app_main_lock(); readers use robot_app_copy(). ISRs never write the mask. | Every buddy routes fault writes through the helper. |
| ISR-shared packed words (encoder, echo) | One GPIO ISR counts encoder edges (≈ 530 edges/s at full speed) and times ECHO (board.c:285-302, 404-482). | ISR writes each packed word with one store; every task-side read-modify-write stays in PRIMASK; no loops added to the ISR; ISR time measured with a GPIO toggle. | B2 and B5 share the ISR time budget. |
| Do task-private working variables count? | PID integrator step ≈ 1.7×10⁻⁵ duty (a tenth of a PWM level); terrain and scan geometry computed in double. | Team position: shared, stored and transmitted data is packed; task-private accumulators keep the precision their resolution needs. Asked the lecturer [mm:ss]: position accepted. | B2 (PID, pose), B4 (height maths), B5 (geometry). |
| Who sets field ranges, and one distance unit | ROBOT_MAX_SPEED_MPS 0.15; HC-SR04 20–4,000 mm; tilt 0.60 rad; hump 5–300 mm; 0.567 mm per count. | Producers sign off their ranges; encoder counts (half-counts for centre path) are the single distance unit; B1 compiles the team data representation table. | B3 widths, B4 height and B5 bypass all use B2's unit. |
| Encoder resolution vs narrowest bar | Narrow bar ≈ 3.0 mm ≈ 5.3 counts; 360 counts/rev only defined for the two-channel config (robot_config.h:78-82). | Not a bit-width issue. B2 measures counts/rev on the real wheel before Week 10. | B3 decode margin depends on B2. |

## Risk Assessment and Subsystem Test Plan

### Integration Risk Register

Enter at least 3 integration risks. Include at least one timing or scheduling risk, one hardware or wiring risk, and one interface or team-integration risk.

| Risk ID | Risk | Cause | Impact | Likelihood | Severity | Mitigation | Owner | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1 | Fault bit lost from an unsynchronised read-modify-write on the shared mask (timing/concurrency) | A low-priority writer (radio P9, Scan P6) is preempted mid-update by Safety/Motion setting another bit | A real stall, tilt or IMU fault is silently cleared; car keeps driving | Low | High | Single locked fault_set()/fault_clear(); stress test toggling WIFI_LOSS/NO_ECHO while stall and IMU bits are injected | B2 + all | Open |
| R2 | Lost encoder counts or false ENCODER fault from the packed ISR word (timing/concurrency) | Task read-and-clear outside PRIMASK; ISR fires between load and store | Stale phase → illegal transition → hard ENCODER stop; or distance error | Low | High | All task access under app_main_lock(); host interleaving test; target test with wheels driven | B2 + B5 | Open |
| R3 | Encoder resolution leaves no margin for the narrowest bar (hardware/sensing) | 3.0 mm bar ≈ 5.3 counts at the 360 counts/rev hypothesis; 5 needed | Barcodes rejected or misread | Medium | High | B2 measures counts/rev and channel mode; lower decode speed if needed; test on the printed sample | B3 + B2 | Open |
| R4 | Firmware and laptop decoder disagree on bit layout (interface) | Layout changed in one place only; byte-order or width mismatch | Wrong telemetry trusted during the demo | Medium | Medium | One wire-format table; schema version in header; shared golden test vectors for C and Python | B1 | Open |
| R5 | Unknown range or saturated value packed as a valid-looking number (interface/safety) | NaN or out-of-range converted without a validity code | Unknown treated as clear space; wrong bypass decision | Medium | High | Explicit 2-bit state on every distance; saturate and flag; host tests that no echo stays non-clear | B5 + B1 | Open |
| R6 | Fixed-point rounding changes decisions at threshold edges (correctness) | Thresholds and tolerances re-expressed in counts / 12-bit / mrad | A barcode, hump or obstacle decision flips | Medium | Medium | Run old double path and new path on the same fixtures and traces; compare events | B3 + B4 + B5 | Open |

### Subsystem Test Plan

Enter at least 4 cross-subsystem tests. Each test must involve the work of at least two buddies and must include a success criterion and evidence type, such as a video, serial log, screenshot, timing result, or commit link. At least TWO test should involve timing, safety, or shared-resource concern.

| Subsystem | Test case | Success criteria | Evidence required | Owner | Status |
| --- | --- | --- | --- | --- | --- |
| Shared fault mask (safety / shared resource) | Toggle WIFI_LOSS and NO_ECHO rapidly while stall, tilt and IMU bits are injected from their tasks | No bit lost or wrongly cleared over 10,000 iterations | Host test log; target serial log | B1 + B2 + B4 + B5 | Open |
| Encoder ISR + Motion (shared resource) | Interleave simulated edges with read-and-clear at every step (host); drive both wheels while forcing extra reads (target) | No count lost, no false edge error | Host log; counts vs reference | B2 + B5 | Open |
| Comm + Motion (safety / timing) | Send binary STOP from the laptop while the wheels are driven | REMOTE_STOP latched; drive inhibited by the existing path; latency not worse than the text version | Serial log + timing before/after | B1 + B2 | Open |
| Telemetry round trip (interface) | Encode boundary values from every producer (speed ±508, pitch ±limit, height 0/300/511, range 20/4,000/no echo, action UTURN, label 15, counters 255) and decode on the laptop | Decoded = source, or saturated value with its flag | Decoder test output; commit link | B1 + B3 + B4 + B5 | Open |
| Decision equivalence (correctness) | Old double vs new packed path on the same barcode fixtures, terrain traces and range thresholds (249/250/251, 599/600/601 mm) | Identical events and decisions | Host test output | B3 + B4 + B5 | Open |
| Before/after evidence (timing) | Measure .bss (map file), stack high-water, message sizes (payload and full MQTT packet), each task step time, GPIO ISR time and masked time of robot_app_copy() | RAM and bytes drop; no deadline miss; ISR and masked time not higher | Week 13 README table | All | Open |

## Constraint Injection / Curveball Response

Instructions • Record the injected constraint exactly as given. • Explain how it affects all five buddies, even if the impact differs by subsystem. • A good response shows prioritisation, trade-off reasoning, fallback behaviour, and evidence-aware decision making.

| Injected constraint | Buddy affected | Immediate impact | Adaptation / compromise |
| --- | --- | --- | --- |
| Change Requirement: Minimum-Bit Data Representation. The robot car firmware must minimise the number of bits used to represent application data. Each data field representing application information must use only the number of bits required to represent its valid range, using C bit-fields or explicit masks and shifts where appropriate (e.g. a value from 1 to 5: 3 bits; a state machine with up to 8 states: 3 bits; fault conditions such as motor stall, encoder fault, IMU fault, no echo and Wi-Fi loss: individual bits within a shared fault bitmask). Applies to motion control and encoder data; line and barcode sensing; obstacle detection; hump estimation; vehicle and controller states; data shared between tasks or between an ISR and a task; inter-task queue messages; and MQTT telemetry and command payloads (compact binary fields, not text). Packed data shared between tasks or with an ISR must be accessed safely, with no read-modify-write races. Week 13: data representation table, RAM before/after, message size before/after, correctness and performance evidence in the GitHub repository. | Buddy 1 — Subque | Telemetry JSON, ASCII commands, STOP routing, IPC buffer sizes, decoder and test tooling all change. ≈ 1.3 KB less static RAM (calculated). Costs: payloads no longer human-readable; decoder mandatory; WIFI_LOSS write joins the shared mask. | Explicit-shift binary frames with a version header; 1-byte commands with a validity pattern; IPC TX_MAX 256 → 20 B, RX_MAX 32 → 1 B; WIFI_LOSS non-stopping under the lock; priority, period and STOP channel unchanged; Python decoder and mosquitto_sub -F %x for debugging. |
| (same) | Buddy 2 — Ong Teck Ang | Encoder ISR word, motion request and snapshot, speed window and fault codes change. ≈ 0.6 KB less static RAM; request 32 → 6 B; snapshot 96 → 16 B. Costs: pack/unpack, sign-extension and saturation risk, strict lock rule, physical re-test. | Pack at boundaries with explicit shifts; accumulators keep precision; ISR words only touched under app_main_lock(); fault bits via the helper; period, priority, gains, STOP and stall policy unchanged. |
| (same) | Buddy 3 — Tan Yi Jian Ethan | ≈ 4.7 KB of optical statics (mostly the codebook) change representation; junction flag becomes a read-modify-write hazard; telemetry ba/bl/bu/br shrink. | 12-bit fixed point; thresholds in ADC counts; widths in counts; flags per producer under lock; action 3 bits, label 4 bits, counters 8 bits; codebook deferred; 2 ms period and distance-based widths unchanged. |
| (same) | Buddy 4 — Dian Ze | terrain_output_t (≈ 72 B) packs to ≈ 12 B; IMU/tilt/collision move into the shared mask; pitch and height risk wrapping. | 12-bit saturating pitch/roll, 9-bit heights, 4-bit motion; mask bits b9–b12 under lock; maths stays double; 10 ms period and 0.60 rad limit unchanged. |
| (same) | Buddy 5 — Siang Hong | Scan storage (≈ 2.4 KB saved), scan results, front range, echo word and handshake change; NaN can no longer mean unknown; mask needs room for NO_ECHO. | Angle-indexed uint16 table with state codes; packed echo word; NO_ECHO non-stopping; completion counters; stop requests moved to stop source; scan timing and 'unknown is never clear' unchanged. |

## Integration Readiness Self-Assessment

Complete all rows. Use Green only if there is evidence, Yellow if partly ready or untested, and Red if the area is unclear or blocking Week 10 integration.

| Readiness area | Status | Evidence | Concern |
| --- | --- | --- | --- |
| Hardware wiring known | Amber | Pin table approved; robot_config.h documents pins, rail and ECHO divider | Car not yet assembled; confirmation flags still 0; encoder counts/rev unmeasured |
| RTOS task ownership clear | Green | app_main.c: P1–P8 application tasks, radio P9; one writer per packed word except the fault mask (locked helper) | None for ownership; timing still unmeasured |
| Shared resources identified | Amber | Fault mask, encoder ISR word, echo word, scan handshake, g_shared snapshot and MQTT IPC buffers identified with writers and locks | Lock discipline for packed words not yet implemented or stress-tested |
| Critical statecharts implementable | Amber | Vehicle 9 states / 4 bits retained; line, terrain and scan states sized | No mission COMPLETE state; production codebook empty |
| Safety or fault handling defined | Amber | 16-bit mask layout, first cause, stop source agreed; stopping vs non-stopping bits defined | Hardware STOP and tilt tests (T1/T5) not yet run |
| Week 10 partial integration feasible | Amber | Changes confined to each owner's files plus robot_app.c commit/copy; host tests exist for every module | Packing work competes with first hardware bring-up; before/after measurements needed |

Updated readiness position before the Week 10 partial integration demo:

IS29 accepts the minimum-bit requirement without changing any task period, priority or safety rule. Every buddy has a sized representation for their data, agreed in one team table, and the fault conditions now share one 16-bit mask with a 4-bit first cause and a 2-bit stop source. The main new risk is concurrency on packed shared words; the team rule is one writer per word, with the fault mask written only through one helper inside app_main_lock(). For Week 10 we will integrate the packed encoder word, motion request and snapshot, fault mask and binary STOP first, because they carry safety, and run the mask stress test and binary STOP test on hardware. Telemetry frames, scan table and barcode widths follow, each proven by an old-vs-new equivalence test. RAM, message-size and timing figures will be measured from the ARM build, not estimated, for the Week 13 evidence. Open items: lecturer confirmation of the barcode symbology, encoder counts per revolution, and the mission COMPLETE state.

Here are the impacts on your Buddy 1 subsystem (Wi-Fi, MQTT, telemetry), ranked from highest to lowest risk. File references are to the public repo.

Ranked impacts

1. Concurrency: race on the shared fault bitmask (HIGH)

- Wi-Fi loss must become a bit in a mask that other tasks also write: Safety, Motion and IMU faults.

- The radio task runs at the lowest priority (P9). If it does mask |= WIFI_LOSS while a higher-priority task sets a stall bit in the same word, one update can be lost. That's a classic read-modify-write race.

- The RP2040's Cortex-M0+ has no LDREX/STREX, so there's no lock-free atomic OR. Every write needs a critical section (PRIMASK) or a single-writer design.

2. Interfaces: telemetry wire format changes completely (HIGH)

- telemetry.h / telemetry.c currently format integer JSON: status up to 255 B every 200 ms, perception up to 252 B and diagnostic up to 144 B every 1 s.

- Binary needs a defined layout: field widths, byte order, signedness, saturation, and a header for version and frame type. Today frames are told apart by JSON key names, and binary has none.

- Avoid sending C bit-field structs as the wire format. Their layout is implementation-defined, so the laptop decoder may not match. Use explicit shifts and masks into a uint8_t buffer.

- A decoder is now required on the dashboard side. Telemetry is no longer human-readable.

3. Other buddies: you depend on their ranges (MEDIUM–HIGH)

- You can't size telemetry fields alone. You need each field's valid range from its owner: wheel speed (motion), pitch and hump height (IMU), range and scan width (ultrasonic), barcode action and label (optical), vehicle state.

- If a range is wrong, values saturate or wrap silently. Agree on one shared data representation table.

4. Commands: ASCII tokens become binary (MEDIUM)

- mqttcmd.c accepts 7 exact tokens up to 32 B. Seven values need only 3 bits, so 1 byte.

- STOP routing in net_mqtt_ipc.c compares 'S','T','O','P' and must change.

- Safety question: is a 1-byte STOP easier to trigger by accident? Consider using the spare bits as a fixed validity pattern.

- Testing changes too: mosquitto_pub -m STOP no longer works, so you need a sender script.

5. RAM: buffers sized for text become oversized (MEDIUM, mostly a gain)

- The IPC TX buffer is 4 × 256-byte messages (~1,040 B static), RX is 4 × 32 B, and net_mqtt.c holds a 257-byte pending copy.

- With frames of about 20 B, these can shrink substantially, likely over 1 KB of .bss saved. Measure it from the map file.

- The char frame[256] buffers are on task stacks. That saving only counts if the stack size is reduced after measuring the high-water mark.

6. Timing / CPU (LOW, likely an improvement)

- JSON formatting loops over decimal digits for every field. Binary packing is a handful of shifts, so the telemetry task should get cheaper.

- Smaller payloads mean less time in lwIP and on air.

- You could consider raising the radio task's priority to make sure Wi-Fi-loss detection is timely.

- Measure the telemetry task's execution time before and after.

Suggested next steps

1. Define the wire format (one table, version header, explicit shifts).

2. Agree field ranges with each buddy.

3. Put every fault-mask write behind one locked helper.

4. Add round-trip encode/decode tests and before/after measurements of RAM, message size and timing.

# Buddy 2: the minimum-bit change, ranked by risk

This is the same analysis as before, written as paragraphs. The firmware hasn't changed since commit `74b8360`. One correction first: the motion output is **88 B on the Pico, not 96 B**. GDB on the firmware ELF shows enums are 1 byte on the Pico (`Tag_ABI_enum_size: small`). 96 B is what the PC host-test build gives, because enums are 4 bytes there. The rest of your premises are correct:

- Motion state is `double` (`core.h:24`).

- The Vehicle→Motion request is 32 B.

- The encoder ISR builds up int32 counts that the 5 ms, priority-2 motion task reads and clears under PRIMASK.

- Everything runs on one core.

On ownership: the handover guide (§3, still a proposal for the team to confirm) gives Buddy 2 `core.c/h`, the encoder and motor parts of `board.c`, and `motion.c/h`. Buddy 5 owns `robot_app.c` and `app_main.c`, Buddy 5 and Buddy 3 share `vehicle.c`, and Buddy 1 owns MQTT. So most changes below need someone else's review. I ranked the risks by chance of breaking × how bad that would be × how many buddies are affected.

**1. Changing `core_real_t` (highest risk).** `core_real_t` is a single global typedef for `double`. `core.h:20-22` refuses to build unless `double` is 64-bit, and `core.c` relies on every int32 encoder count fitting exactly. Other buddies use it too. Buddy 4's hump-height maths, Buddy 5's ultrasonic range, range-to-point and stopping-clearance functions, Buddy 3's optical odometry (`robot_app.c:1352`) and the vehicle's pose anchors all depend on it. Shrinking the typedef would silently change all of their maths at once. The safer route is to keep `double` for computing and add motion-specific packed types only where data is stored, shared or sent. Make that a team decision first, because Buddy 4 and Buddy 5 review `core.c`.

**2. Precision loss in odometry and PID (high risk).** One encoder count is 0.567 mm of travel and 4.05 mrad of yaw. The motion code must still resolve a 5 mm distance tolerance, a 0.03 rad turn tolerance and a 12 mm/s stationary threshold. Pose x/y is integrated every 5 ms. If each step is rounded, the error has the same sign every step at constant heading, so it adds up to steady drift instead of averaging out. The speed window (`motion.c:356-407`) splits fractional counts across buckets, so integer storage needs fractional bits there. The motion and core test suites check results to 1e-10, so they will need reworking. The fix is to keep distance, centre path and yaw as integer encoder-count sums, which are exact and small. Only x/y need a wider internal accumulator, rounded when stored or published.

**3. Narrow encoder counters in the ISR (high risk).** Today an int32 delta never realistically saturates, but any edge error latches a hard ENCODER fault (`robot_app.c:282-285`). With an 8-bit delta, the limit is ±127 counts, which is 72 mm of wheel travel between reads. Nominal 5 ms reads stay far below that. Start-up is different: the ISR is enabled in `board_init` before any task runs, and someone may push the robot by hand. Saturating then causes a false hard fault, and forgetting to saturate lets the value wrap silently into wrong odometry. A second hazard: if the ISR's private quadrature phase bits share a word with the deltas, a task-side `word = 0` clear wipes them, and the next edge is decoded wrongly. Size the fields from the measured physical top speed × the worst gap between reads, saturate them, and clear with a mask or keep phase in its own word.

**4. Race-free access to packed data (medium-high risk).** The RP2040's Cortex-M0+ has no LDREX/STREX instructions. So no read-modify-write on a packed word can be atomic without disabling interrupts, and PRIMASK (`app_main_lock`) stays the only tool. Bit-fields make this easy to miss, because writing one field rewrites its whole container word. Any write made outside the lock can therefore wipe another writer's update. The current code is safe:

- Every runtime write to `g_shared` sits inside one of the 19 locked regions.

- The encoder read-and-clear is locked.

The rule to keep after packing is:

- Every write to shared packed data happens inside the lock.

- Packing, unpacking and float work happen outside it.

- Fields from different writers never share a word, such as encoder data next to Buddy 5's sonar data.

There is also an upside. Every task copies all 496 B of `g_shared` with interrupts masked, three of them every 2 ms. A smaller `g_shared` shortens those windows and lowers encoder-interrupt latency.

**5. The Vehicle→Motion request (medium risk).** The request can shrink a lot:

- `kind` fits in 2 bits.

- Forward speed in mm/s fits in 9 signed bits (limit 0.15 m/s).

- Yaw rate in mrad/s fits in 11 signed bits (limit 1.0 rad/s).

- A turn target in mrad fits in 14 signed bits.

But `command_id` uses wrap-around ordering over 2³², and 0 is reserved (`motion.c:99-130`). With N bits, an old ID is only recognised as old within the last 2^(N−1) IDs, so the constants and tests must change. `request_version` is compared with `!=` (`robot_app.c:296`). If it is too narrow, exactly 2^N publishes between two reads bring it back to the same value and a new request is missed, so use at least 4 bits. `vehicle.c` belongs to Buddy 5 and Buddy 3, with Buddy 2 reviewing the motion requests.

**6. Motion-output consumers and the fault bitmask (medium risk).** Several readers depend on the 88 B motion output:

- The vehicle reads the pose.

- Buddy 4's terrain code reads wheel speeds and `b_stationary`.

- Buddy 5's scanner reads `b_stationary` and the front trigger pose.

- Buddy 1's telemetry sends speeds in mm/s.

- Reset checks `b_stationary`.

Every new encoding ripples into their steps. Faults are stored today as a first-fault code plus a hard flag. The 15 reasons fit a 16-bit mask, and motion raises 5 of them: encoder, motion, left stall, right stall and motor output. The brief's "no echo" and "Wi-Fi loss" are not fault reasons yet, so Buddy 5 and Buddy 1 must add them. Telemetry and reset still expect a single code, so keeping a 15-bit active mask plus a 4-bit first-fault index preserves both meanings. This stays race-free because `robot_app_fault` already updates under the lock.

**7. Timing and CPU (medium risk, not yet measured).** All `double` maths runs in software. The map shows libgcc's `__aeabi_dadd`, `dmul` and `ddiv`, and the M0+ has no FPU. There are two ways to build the change:

- **Hybrid:** compute in `double` and pack only at the boundaries. This adds conversions every 5 ms and costs slightly more CPU, but it is the safest.

- **Full fixed-point:** probably much faster, but it carries the risks in point 2.

Either way, motion runs at priority 2, so any extra time delays the line and scan tasks. The safety task treats line data older than 20 ms as a hard fault. A narrowed `dt` (15 bits of µs covers the 25 ms limit) must saturate rather than wrap, because a wrapped long gap looks short and corrupts the speed. 16-bit timestamps fit the 50 ms lease and 15 s timeout, but only if every comparison is masked the same way. Passing integer PWM levels (a signed 13-bit value, capped at 2187) to the motor code would remove its float work. Measure the worst-case motion step with `board_time_us()`, because the M0+ has no cycle counter.

**8. Encoder totals (low-medium risk).** The 32-bit wrap-around wheel totals feed Buddy 3's line task every 2 ms, which works out the difference using a 2³¹ half-range. Twelve bits would still allow ±1.16 m of travel between reads. However, the half-range constant and the masked subtraction would change, and that code is Buddy 3's.

**9. RAM (low risk).** Most of the saving is in `motion_t` (592 B), especially the 272 B speed window and the 128 B config. There is also a hidden saving: every task keeps a local copy of `g_shared` on its stack, so shrinking it lowers all eight stack high-water marks. The traps are in measuring:

- PC builds give different sizes (96 vs 88 B, 600 vs 592 B), so measure only the ARM build.

- The 8 × 4096 B task stacks live outside `.bss`.

- Stack savings only show up as lower high-water marks unless the stack size itself is reduced.

**10. MQTT (low risk for Buddy 2).** The binary frames are Buddy 1's job, replacing today's JSON. Buddy 2 supplies the range, unit, resolution and bit width of each motion field. Those are also your rows in the data representation table.

Before changing anything, tag the current commit and record the "before" figures from the ARM build:

| Item | Before |

|---|---|

| Request | 32 B |

| Output | 88 B |

| `g_motion` | 592 B |

| `g_shared` | 496 B (rebuild to confirm) |

| `.bss` / `.data`, offline build | 23,392 / 328 B |

| Stack frames | `motion_step` 200 B, `core_pid_step` 112 B, `core_odometry_step` 96 B |

Four things are still unverified:

- The motion step's execution time, which needs the robot.

- The physical top wheel speed, which you need to size the encoder fields.

- The stack frame of `robot_app_motion_step`, which needs a full build.

- Whether internal `motion_t` state counts as "application data". Ask the lecturer, because that answer decides between the hybrid and full fixed-point designs.

Not quite all of it is double. Raw ADC readings are uint16_t, timestamps are uint32_t and flags are bool. But darkness, confidence, steering, bar widths and codebook template widths are double, and that's most of the RAM.

Impacts, ranked by risk

1. Concurrency: losing a junction edge (High)

- b_junction_edge is set by LineTask every 2 ms and cleared by Vehicle every 10 ms (robot_app.c:358, :412).

- Today it's a separate bool, so each write is a single store. If it's packed into a word other tasks also write, every update becomes read-modify-write: a task can overwrite the whole word with a stale copy and lose the edge. The result is a missed turn, then the pending action times out into FAULT after 10 s.

- Fix: every write to a shared packed word goes inside app_main_lock(), with one flag word per producing task.

2. Accuracy: rounding changes decode results (Medium–High)

- Tolerances are 15% relative and 0.3 mm absolute (barcode.c:51-52). Re-expressed in encoder counts, 0.3 mm is less than one count, so it must become at least 1 count.

- Converting the 0.75/0.55 black thresholds to integers can move decisions at the edges.

- Fix: keep the current double decoder as a reference in the host tests, and check both versions give identical events on the same test barcodes.

3. Accuracy: no margin on the narrowest bar (Medium–High, not caused by packing)

- In the Barcode Sample a narrow bar is about 3.0 mm, about 5.3 counts at 0.567 mm per count. The rule needs at least 5 readings per bar.

- This passes with almost no margin either way. Bit width doesn't change it; encoder resolution does.

4. RAM: large saving (Low risk, high value)

Sizes are sizeof from the ARM compiler for the Pico:

┌───────────────────────────────────────────────────┬────────┬────────────────────────────────────────────────────────────────┐

│                     Structure                     │  Now   │                       After (calculated)                       │

├───────────────────────────────────────────────────┼────────┼────────────────────────────────────────────────────────────────┤

│ Codebook (barcode_config_t, 16 symbols × 32       │ 4,344  │ ~100 B if stored as one narrow/wide bit per element (Code 39   │

│ doubles)                                          │ B      │ only)                                                          │

├───────────────────────────────────────────────────┼────────┼────────────────────────────────────────────────────────────────┤

│ Barcode state (32 double widths)                  │ 320 B  │ ~60 B with 8-bit counts                                        │

├───────────────────────────────────────────────────┼────────┼────────────────────────────────────────────────────────────────┤

│ Line output                                       │ 48 B   │ ~9 B                                                           │

└───────────────────────────────────────────────────┴────────┴────────────────────────────────────────────────────────────────┘

- 12-bit darkness and widths in encoder counts lose nothing the sensors can resolve, because the ADC is 12-bit and the encoder moves in whole counts.

- The line output is copied by every task in robot_app_copy(), so a smaller one makes every copy faster.

5. Timing: LineTask likely faster (Low)

- The M0+ has no floating-point hardware, so each double operation runs as slow software code, every 2 ms.

- Precomputing thresholds in ADC counts removes the per-sample divide. Steering still needs one divide, which the RP2040's hardware divider handles.

- Shifts and masks cost a few cycles. This still needs measuring before and after.

6. Minimum widths per field

┌────────────────────────┬─────────────────────

│ Line status (6 states) │ 3                       │

├────────────────────────┼─────────────────────────┤

│ 9 flags                │ 1 each                  │

├────────────────────────┼─────────────────────────┤

│ Darkness               │ 12                      │

├────────────────────────┼─────────────────────────┤

│ Steering (shared)      │ 8–12                    │

├────────────────────────┼─────────────────────────┤

│ Bar width              │ 8 (counts)              │

├────────────────────────┼─────────────────────────┤

│ Run count (0–32)       │ 6 (33 values, so not 5) │

├────────────────────────┼─────────────────────────┤

│ Barcode action         │ 3                       │

├────────────────────────┼─────────────────────────┤

│ Barcode label          │ 4 (codebook index)      │

├────────────────────────┼─────────────────────────┤

│ bu / br counters       │ 8, saturating at 255    │

└────────────────────────┴─────────────────────────┘

Timestamps should stay 32-bit: they're a shared time base, and the wrap-safe subtraction relies on it.

Other buddies

┌─────────────────────┬────────────────────────────────────────────────────────────────────────────────────────────────────────┐

│        Buddy        │                                                 Impact                                                 │

├─────────────────────┼────────────────────────────────────────────────────────────────────────────────────────────────────────┤

│ B1 (comms)          │ ba, bl, bu and br move from text to binary. Agree the widths (action 3, label 4, counters 8). OPTICAL  │

│                     │ could use two fault bits: bad sample vs second barcode.                                                │

├─────────────────────┼────────────────────────────────────────────────────────────────────────────────────────────────────────┤

│ B2 (motion)         │ Steering resolution he can accept (used at robot_app.c:984). Confirm counts/rev, because bar widths    │

│                     │ become encoder counts.                                                                                 │

├─────────────────────┼────────────────────────────────────────────────────────────────────────────────────────────────────────┤

│ B4 (terrain)        │ Hump height uses the same distance, so agree one unit.                                                 │

├─────────────────────┼────────────────────────────────────────────────────────────────────────────────────────────────────────┤

│ B5                  │ Lock rule for flags packed next to their scan flags. Reacquisition only reads status == TRACKING, so 3 │

│ (obstacle/vehicle)  │  bits is enough for them.                                                                              │

└─────────────────────┴────────────────────────────────────────────────────────────────────────────────────────────────────────┘

An alternative to consider and probably reject

You could measure bar widths as LineTask sample counts (time) instead of encoder distance. That needs fewer bits and doesn't depend on the encoder. But time depends on speed: a slower pass looks like a wider bar. Distance is why the decoder works at all, so I don't recommend it.

What to verify before accepting this

- Struct sizes against the map file from a real build.

- LineTask time before and after, measured under full load.

- Counts/rev with Teck.

- Code 39 with the lecturer, before shrinking the codebook.

Impact of minimum-bit data representation on the IMU / terrain subsystem (Pico W, µT-Kernel, single core), ranked by risk:

R1 — Concurrency: shared fault bitmask read-modify-write (highest, safety).

IMU, TILT, COLLISION and TILT_UNAVAILABLE currently take separate fault paths. Folding them into one shared mask makes each update a read-modify-write (mask |= IMU_FAULT). Your IMU task (10 ms) shares that word with the Safety task (2 ms) and the GPIO ISR, and the Cortex-M0+ has no LDREX/STREX, so there is no lock-free atomic OR — a preempted RMW drops another writer's bit and a real hard fault silently disappears. Route every write through the existing PRIMASK app_main_lock() via one fault_set()/fault_clear().

R2 — Correctness/safety: packing pitch and height.

Pitch and the four height fields are doubles. Packing adds quantisation and a saturation boundary — the signed pitch field must saturate at the 0.60 rad tilt limit, not wrap, because a wrapped angle past the limit could read as level and miss a tilt fault. Motion state is 9 values → 4 bits, not 3; hump height is 5–300 mm → 9 bits, not 8 (0–255 can't hold 300). Size widths from measured resolution; height is still a ~14 mm model.

R3 — Timing/CPU: two opposing effects, net unmeasured.

The M0+ has no FPU, so signed path × sin(pitch) is software-emulated every 10 ms — one idea is to fixed-point the height integration itself to save cycles. Against that, each locked mask write adds a masked-interrupt window competing with the ISR and your I2C busy-wait (~1.2 ms normal, up to ~8 ms on timeout — estimates, not measured WCET). Net must be measured, not asserted.

R4 — RAM (positive, modest).

terrain_output_t ≈ 72 B (7 doubles + 9-value enum + counter + 6 bools) → ~12 B packed. g_shared is copied by every task via robot_app_copy, so a smaller terrain block shortens every copy. Confirm with sizeof + the .map file.

R5 — Interfaces.

Inputs from Motion (signed_distance_m, encoder_yaw_rate_rps) and outputs to Safety/Vehicle and to Comm (binary MQTT) all change layout. Bit positions and field widths must be agreed, especially the shared fault word.

R6 — Other buddies.

Teck (packed path/yaw format + consistent snapshot; ISR-vs-task fault writes), Subque (telemetry field widths; same fault word; Wi-Fi bit), Siang Hong (no-echo bit shares the mask — fix bit positions), Ethan (shares the g_shared copy). One fault_set/clear macro, agreed bit positions, every write under the lock.

B5 impacts of Change Requirement 1, ranked by risk

I read all three files you named. Some impacts land in robot_app.c and vehicle.c, so I cite those too. Every size below comes from struct layouts (Cortex-M0+, where doubles align to 8 bytes), not from a map file.

1. Unknown distance loses its meaning (interface, safety): high

- What: today NaN means "unknown". scanner_result_reset sets all four result distances to NaN (scanner.c:230-238). A failed capture also stores NaN (scanner.c:259-263). An integer millimetre field can't hold NaN.

- Failure: a careless conversion turns unknown into 0 mm, or into some plausible value. 0 mm is a phantom obstacle under the 0.25 m stop. A large value reads as clear space.

- Downstream: robot_app.c:1025-1028 builds the bypass offset and length from apparent_width_m and nearest_distance_m. Today a NaN there gets blocked by b_obstacle_seen (robot_app.c:1033-1036). With integers, width = 0 produces a believable offset, so that gating must survive the change.

- Mitigation: every packed distance carries its validity: the range state for samples, the existing b_*_observed and b_obstacle_seen flags for results. Add a host test showing unknown and out-of-range stay "not clear" through pack and unpack.

2. An angle-indexed table breaks refine and the profile if "not sampled" leaks (interface, behaviour): high

- What: storing one entry per angle (61 even angles from -60 to +60) is the obvious minimum-bit design. But three places treat any non-VALID entry as unknown:

- scanner_refine flags it (scanner.c:302-312),

- scanner_accumulate adds it to unknown_samples (scanner.c:396-399),

- the left and right sector validity checks (scanner.c:386-395) clear their valid flag on it.

- Failure: after a coarse pass, 48 of the 61 entries were never sampled. Refine would then sweep the whole ±60°, and b_profile_valid would never be true (scanner.c:364-365). So b_bypass_clearance would never be true (robot_app.c:1036), and the robot could never bypass.

- Mitigation: add a fourth state, "not sampled", which all three loops skip. Boundary tests at -60, -2, 0, +2, +60. The existing scanner host tests must pass unchanged. If that fails, the fallback is a packed 32-bit sample list.

3. Packing flags with two writers causes lost updates (concurrency): high severity, medium likelihood

- What: the Scan task (priority 6) sets b_coarse_ready (robot_app.c:1102) and b_fine_ready (robot_app.c:640). The Vehicle task (priority 4) clears both (robot_app.c:413-414). The new fault mask will also have several writers.

- Failure: pack them into one byte, and one task's read-modify-write can be preempted by the other. The interrupted task then writes back a stale byte, and a cleared bit comes back. Vehicle then acts on a scan that never finished, or waits until the scan timeout.

- Today: safe, because they're separate fields and every write is inside app_main_lock().

- Mitigation: every write to a packed shared word stays locked. Better still, give each word a single writer: Scan bumps a small completion counter, and Vehicle remembers the last value it acted on.

4. Echo capture shared between the ISR and the task (concurrency, timing): medium

- What is actually shared:

- g_echo_state: the ISR writes it (board.c:812-841). The task writes it in trigger, service and result (board.c:416-427, 451-455, 469-479).

- g_echo_pulse_us: the ISR writes it, the task reads it.

- g_echo_rise_us is ISR-only (board.c:816, 823), so it isn't a shared-data issue. Cutting it to 32 bits is a size change only.

- Platform: the Cortex-M0+ has no LDREX/STREX, and no second core is used (no multicore anywhere in the firmware). A single aligned 32-bit load or store is atomic. A read-modify-write is not.

- Mitigation: pack state (3 bits) and pulse (15 bits) into one uint32_t, so the ISR publishes both with one store and the task can't see a half-updated pair. Task-side read-modify-writes must stay under app_main_lock(), which is PRIMASK (app_main.c:63-71). The risk is someone removing the lock because "it's one word now".

- Timing: the echo shares board_gpio_isr (board.c:758) with the encoder edges, so any extra ISR work delays encoder counts. Keep it to one store, measure ISR duration before and after, and agree a budget with B2.

5. NO_ECHO fault bit: what it means and whether it fits (interface): medium

- Semantics:

- An HC-SR04 also gives no echo when nothing is within range, so NO_ECHO describes a condition, not a broken sensor. It must not stop the car on its own.

- Safety already comes from "unknown range never grants forward motion": FOLLOW halts into RANGE_WAIT (vehicle.c:527-536), and RANGE_WAIT faults after range_wait_timeout_ms = 1,000 (vehicle.c:224-228, app_cfg.c:52).

- Definition to agree: board_ultrasonic_result gives b_valid = false

6. Decision boundaries after millimetre rounding (correctness): low–medium

- The comparisons are <= 0.60 m (scanner.c:304, 417) and <= 0.25 m / > 0.25 m (robot_app.c:1019-1022).

- Pick one rounding rule and use integer thresholds everywhere. Test 249/250/251 mm and 599/600/601 mm against the current double build.

- Accuracy isn't the issue: rounding to 1 mm adds at most 0.5 mm, below what the sensor resolves.

7. CPU and timing: low risk, likely a gain

- Faster profile: scanner_latest is an O(n²) newest-per-angle search (scanner.c:328-341), called for every sample from scanner_profile (scanner.c:357-363). That's up to 74 samples (scanner.c:267). An angle-indexed table removes it.

- Unpacking is cheap: it costs a few shifts per entry, once per scan. The M0+ has no FPU, so the soft-float sin() per obstacle beam (scanner.c:420-421) already costs far more. Keep geometry in double, in local variables.

- Shorter masked window: every task copies all of g_shared with interrupts off (robot_app.c:766-770), and Safety, Line and Scan do it every 2 ms. A smaller struct means a shorter window and less jitter on encoder and echo edges.

- No change needed to scan priority 6, the 2 ms period, 65 ms trigger spacing, 30 ms echo timeout or 180 ms settle (robot_config.h:120-123). Packing changes storage, not scheduling.

8. RAM: low risk, but report it honestly

- scanner_state_t.samples is 80 × 32 B = 2,560 B of static .bss (scanner.h, g_scanner at robot_app.c:81). An angle-indexed uint16_t table is 61 × 2 = 122 B.

- scanner_result_t is about 56 B. Its fields need 66 bits.

- Stack: each task has a fixed 4,096 B (app_main.c:18). A smaller g_shared copy saves nothing unless the stacks are resized after a high-water measurement.

- Open question for the lecturer: does "every application data field" include config such as the doubles in scanner_config_t (scanner.h), or only runtime data?

Who else is affected

Check before accepting

- Fault-mask layout, NO_ECHO definition, and a timing budget for the shared GPIO ISR: all three need agreement with other buddies.

- Every size here is a layout estimate. The Week 13 numbers have to come from the map file, stack high-water marks and GPIO-toggle timing.

A note for you, off screen:

- Your drafts list g_echo_rise_us as shared between the ISR and the task. It's ISR-only.

- Point 2 above is a sharper version of the "not sampled" check in your P3 prompt. Make sure you can explain it in your own words before the tribunal.